## Ordering Info: NH265-16

## NH265-16 Technical Specification

| PCI-E | PCI-Ex1 2.0 compliant |
| --- | --- |
| Dimensions(mm) | 175 ×102 |
| Power consumption | < 9.4w |
| Video compression | H.264, NTSC/PAL auto-detect |
| Number of video input | 16 |
| Video input connector | BNC(1.0Vp 1.0Vp，75Ω) |
| Preview resolution | D1 |
| Recording resolution | CIF/D1 |
| Encoding FPS per channel | 1~25 PAL, 1~30 NTSC |
| Encoding FPS per card | When D1 16-channels, max 300 FPS per card |
| Audio compression | MPEG audio |
| Number of audio input | 16 |
| Audio input connector | BNC(2.0Vp 2.0Vp 2.0Vp-p，1000Ω ，audio volume control range 83DB) |
| Audio sampling rate | 16kHZ |

## Features

- PCI-E port allows more bandwidth for fluent data transfer.

- Hardware compression allows low CPU utilization.

- Excellent live preview and recorded video quality.

- Reliable and stable suitable for long time operation.

- High performance with low power consumption.

- Real-time video and audio synchronized compression without dropping frames.

- Configurable encoding picture type (I frame, P frame, etc)

- Configurable picture quality and data bit-rate.

- Supports hardware motion detection.

- Support OSD with all Unicode characters support (including traditional & simplified Chinese)

- Support adjustment of image brightness, hue, saturation, and contrast.